| Roll No: 160123733174 Exp. No: 08 Date: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| EXPERIMENT-8 - Aim: To connect two PCs in different networks using routers and enable communication through static routing. Description: - Two routers connect two different networks. - PC0: 192.168.1.2/24, GW: 192.168.1.1. - PC1: 192.168.2.1/24, GW: 192.168.2.1. - Routers are linked with an inter-network connection and configured with static routes for communication. Diagram: Table: |  |  |  |  |  |
|  | Device | IP address | Subnet | Gateway |  |
|  | PC0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |  |
|  | PC1 | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |  |
| Procedure: - Connect two PCs to routers and link routers. - Assign IPs: - Router0 LAN: 192.168.1.1/24 - Router1 LAN: 192.168.2.1/24 - Inter-router link: 10.0.0.1/30 & 10.0.0.2/30 - Enable interfaces (no shutdown). - Configure static routes: - R0: ip route 192.168.2.0 255.255.255.0 10.0.0.2 - R1: ip route 192.168.1.0 255.255.255.0 10.0.0.1 - Test with ping. Result: CLI console message: |  |  |  |  |  |
| Page No. 45 Signature of the Faculty………………………... |  |  |  |  |  |

| Roll No: 160123733174 Exp. No: 08 Date: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Conclusion: PCs in different networks can communicate via routers using proper IP configuration and static routing. 2)Aim: To design and configure a network of multiple PCs connected through hubs, switches, and routers, and enable communication between different networks using static routing. Description: - Two networks are connected via Router0 and Router1. - Each router connects to a switch, and switches connect to hubs with multiple PCs. - PCs in the left network: 192.168.1.x/24. - PCs in the right network: 192.168.2.x/24. - Routers use an interconnection link for communication and static routing is used to share routes. Diagram: Table: |  |  |  |  |  |
|  | Device | IP address | Subnet | Gateway |  |
|  | PC0 | 192.168.1.2 | 255.255.255.0 | 192.168.1.1 |  |
|  | PC1 | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |  |
|  | PC2 | 192.168.1.4 | 255.255.255.0 | 192.168.1.1 |  |
|  | PC3 | 192.168.1.5 | 255.255.255.0 | 192.168.1.1 |  |
|  | PC4 | 192.168.2.2 | 255.255.255.0 | 192.168.2.1 |  |
|  | PC5 | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 |  |
|  | PC6 | 192.168.2.4 | 255.255.255.0 | 192.168.2.1 |  |
|  | PC7 | 192.168.2.5 | 255.255.255.0 | 192.168.2.1 |  |
| Procedure: - Topology Setup: Connect PCs → Hubs → Switches → Routers, and link both routers. - Assign IPs: - Left PCs: 192.168.1.2 – 192.168.1.5, GW: 192.168.1.1. - Right PCs: 192.168.2.2 – 192.168.2.5, GW: 192.168.2.1. |  |  |  |  |  |
| Page No. 46 Signature of the Faculty………………………... |  |  |  |  |  |

Roll No: 160123733174	Exp. No: 08	Date:

- Router-to-Router link: 10.0.0.1/30 & 10.0.0.2/30.

- Configure Routers: Assign IPs to all router interfaces, run no shutdown.

- Static Routes:

  - On Router0: ip route 192.168.2.0 255.255.255.0 10.0.0.2.

  - On Router1: ip route 192.168.1.0 255.255.255.0 10.0.0.1.

- Testing: Ping from any PC in 192.168.1.x to any PC in 192.168.2.x.

Result:

CLI console message:

Conclusion:

The network allows multiple PCs from two different LANs to communicate via routers using switches, hubs, and static routing. Proper IP addressing and routing ensure successful inter-network communication.

Page No. 47	Signature of the Faculty………………………...